ЕС-SAFETY DATA SHEET

ISOLLAT-03

- IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND THE COMPANY/UNDERTAKING

Trade name 							          Isollat-03

Information about 	Special Technologies LLC

manufacturer	39/35, Chapaev Str., 623704, Berezovskiy,

Sverdlovsk Region, Russia

Fax: (343) 345-27-26

Phone:	 (343) 345-27-27

Phone:	 (343) 345-27-28

- COMPOSITION / INFORMATION ON INGREDIENTS

Description

| Components | CAS-No. | Weight % |
| --- | --- | --- |
| Acrylic acid compound ester | 25085-34-1 | =>35 |
| Soda lime borosilicate glass | 65997-17-3 | =>65 |

3.    HAZARDS IDENTIFICATION

No special hazards known.

Irritation may occur in case of  adherence of the product to skin and drying.

4.    FIRST AID MEASURES

IF INHALED:								 remove victim to fresh air and

provide medical attention.

IF ON SKIN:	Wash with water and soap.

IF IN EYES:	 Flush the affected eye thoroughly

with plenty of water, preventing

penetration of the substance into

the unaffected eye. If irritation persists,

seek physician advise.

IF SWALLOWED:	 Rinse mouth immediately with water and

drink plenty of water,

get medical attention.

ЕС-SAFETY DATA SHEET

ISOLLAT-03

5.    FIREFIGHTING MEASURES AND FIRE SAFETY DEVICES

Suitable extinguishing media 	Water spray, foam,

carbon dioxide,

extinguishing powder

Unsuitable extinguishing media	None known.

for safety reasons

Safety precautions in case of fire                                                	Do not breathe carbon monoxide (СО)

6.   ACCIDENTAL RELEASE MEASURES

6.1. Personal precautions, protective equipment and emergency procedures

Ventilate the room.

6.2. Environmental precautions.

Prevent from penetration into the environment.

6.3. Methods and materials for containment and cleaning up.

For small spills: Collect using appropriate liquid-binding materials (sand, sawdust, etc.). Dispose the absorbed substance according to applicable regulations. For large spills: Pump out. Wash residues with water.

7.    HANDLING AND STORAGE

7.1.  Handling

Observe conventional safety precautions for handling with chemical substances.

7.2. Storage

Storage temperature from +00С to +250С.

8. WORKPLACE EXPOSURE CONTROLS AND PERSONAL PROTECTION

8.1. Parameters subject to control

Components with specified occupational exposure limits

| Components | CAS-No. | Limit type | Value | Remark |
| --- | --- | --- | --- | --- |
| Ammonia | 7664-41-7 | CLV | 20 mg/m3 |  |

8.2.  Personal protective equipment

Eye/face protection                                                                           Safety glasses with side-shields (EN 166).

Skin/hands protection                                                              Wear appropriate chemical

resistant 	gloves (EN 374).

8.3.  Emergency telephone number:  (343) 345-27-27,  working hours from 800 a.m. to 500 p.m.

ЕС-SAFETY DATA SHEET

ISOLLAT-03

9.   PHYSICAL AND CHEMICAL PROPERTIES

Physical state                                                                       	  Liquid

Color	  White

Odor                                                                                           Mild

Information on: Water freezing point	  00С

Information on: Water boiling point	  1000С (DIN ISO 976)

Flash point	  Not applicable

Information on: Water vapor pressure	  23 mbar (at 200C)

Density                                                                                       Not exceeding 0.5 g/cm3

Water solubility	  Partially soluble (at 150С)

Miscibility with water	  Miscible

Brookfield apparent viscosity	  50000-80000 cP  (rotor R4, rotation

3 min-1, at temperature 230С)

10.   STABILITY AND REACTIVITY

10.1. Reactivity

This material is considered to be non-reactive under normal conditions of use.

10.2. Chemical stability

Stable

10.3. Possibility of hazardous reactions

No hazardous polymerization observed.

10.4. Conditions to avoid

Excessive heat. Freezing.

11.  TOXICOLOGICAL INFORMATION

LD50 Rat (oral): >2,000-10,000 mg/kg

Irritant effect evaluation: In case of adherence of the product to skin and drying, irritation may occur. Primary skin irritation, rabbit: Not irritative (OCDE, Directive 404). Primary mucous membrane irritation, rabbit: Not irritative (OCDE, Directive  405).

According to the experience and data available, in case of proper handling and usage following the specified rules, there are no harmful effects on health known. This statement is made based on the data for products with similar composition.

12.   ECOLOGICAL INFORMATION

When introducing low concentrations into biological sewage disposal plants in accordance with applicable instructions, the product does not affect purification ability of activated sludge.

Evaluation of bio-degradability and eliminability (H2O): Significant amounts of the product can eliminate from water as a result of abiotic processes, e.g. adsorption by activated sludge.

Bioaccumulative potential: Polymeric part due to its structural properties is not bio-available. No accumulation in living organisms occurs.

Prevent the product from penetration into water bodies without appropriate treatment. Based on the information currently available adverse effects on environment are not likely to occur.

ЕС-SAFETY DATA SHEET

ISOLLAT-03

13.  DISPOSAL CONSIDERATIONS

The product shall be delivered to the specifically fitted dump/place for storage of waste and refuse or to the specially equipped waste incineration plant.

14.  TRANSPORT INFORMATION

Overland transport

ADR

According to transport regulations the product is not classified as dangerous goods.

RID

According to transport regulations the product is not classified as dangerous goods.

River transport

ADNR

According to transport regulations the product is not classified as dangerous goods.

Marine transport

IMDG

According to transport regulations the product is not classified as dangerous goods.

Air transport

IATA/ICAO

According to transport regulations the product is not classified as dangerous goods.

15.  REGULATORY INFORMATION

ЕС Regulation No. 1907/2006, of the European Parliament and Council of December, 18th, 2006 on Registration, Evaluation, Authorization and Restriction of Chemicals.

Resolution of the European Parliament and Council (EC) No. 1272/2008 of 20.01.2009 on the Classification, Labeling and Packaging of Substances and Mixtures.

16. OTHER INFORMATION

Other fields of applications shall be agreed with the manufacturer.

Information in this safety data sheet is based on knowledge and experience available at the present day, and it describes the material in terms of safety requirements. This information shall not be considered as a description of the product's properties (product specification). There shall be no conclusions made with regard to quality and suitability of the product for any specific application based on the information in this safety data sheet. Its the responsibility of the end user to observe all existing laws and regulations as well as statutory standards.